- How to draw “Branch Line Coupler”

Assumed that:  Z0 = 50 Ω has transmission line width 2.42 mm

Z0/√2 = 35.35 Ω has transmission line width 3.95 mm

λ/4 = 30mm

50 Ω transmission line for port connection, L = 5 mm and W = 2.42 mm

- So we can draw the “Branch Line Coupler” on AutoCAD as below:

- Draw 50 Ω transmission line with width and length above then copy and paste to get two stub.

- Draw 35.35 Ω transmission line with width and length above then copy and paste to get two stub.

- Select the stub that you want to move then type “m” + “Enter” and move from the center point of the width.

(1)                                           (2)                                                                  (3)

- Draw 50 Ω transmission line for port connection (5mm)

- To cut the cross line we use comment: “tr” + “Enter” then click on the line that you want to remove.

(4)

- Draw the PCB border (use Poly line).

- To reduce the etching time we can hatch some part of copper on PCB except circuit part. Noted that, black part will remove during etching process.

- To hatch the remove part you can you comment: “h” + “Enter”, chose the “Black” color.

- The remove part along the transmission line (width of black part) must be greater than 1mm.

(5)                                                                        (6)

- Select all the lines on the PCB then remove them.

- How to draw “Ring Hybrid”

Assumed that:  Z0 = 50 Ω has transmission line width 2.42 mm

Z0√2 = 70.71 Ω has transmission line width 1.7 mm

λ/4 = 30mm

50 Ω transmission line for port connection, L = 5 mm and W = 2.42 mm

- The circumference of the reference circle is 6λ/4 = 180 mm, so the radius r = 28.66 mm. Let draw a circle with r1 = r-1.7/2 =28.66 mm - 0.85 mm = 27.81 mm.

- Draw a new circle with r2 = r+1.7/2 =28.66 mm + 0.85 mm = 29.51 mm.

- Draw the reference line on the circle.

- (2)                                               (3)

- Draw the stub with length, L = r2 + 5mm=34.51mm and width, W = 2.42 mm from the center point.

- Copy and paste that stub to get 3 more. Rotate 2 of stub with 60 degree and 120 degree.

(4)                                                                     (5)

- Move all start points of the stubs to the center point of the circle.

(6)

- Draw the PCB border (use poly line) and the area that you want to remove copper.

- Use comment “tr” + “Enter” to remove cross line.

(7)

- Hatch the remove part. “h” + “Enter”

- Remove all line on pattern.